// Simple Temperature ALarm using LM35 and an active buzzer.

const int LM35_PIN = A0;

const int BUZZER_PIN = 8; //active buzzer

const int LED_PIN = 13;

float thresholdC = 30.0;

unsigned long readInterval = 1000;

unsigned long lastRead = 0;

void setup (){

Serial.begin(9600);

pinMode(BUZZER_PIN, OUTPUT);

pinMode(LED_PIN,OUTPUT);

digitalWrite(BUZZER_PIN ,LOW);

digitalWrite(LED_PIN, LOW);

Serial.println("LM35 Temperature Alarm Starting...");

}

void loop (){

unsigned long now = millis ();

if (now - lastRead >= readInterval){

lastRead = now;

int raw = analogRead(LM35_PIN);

float voltage = raw * (5.0/1023.0);

float tempC = voltage / 0.01;

if(tempC >= thresholdC) {

//Alarm ON

digitalWrite(BUZZER_PIN, HIGH);

digitalWrite(LED_PIN,HIGH);

}else{

// Alarm OFF

digitalWrite(BUZZER_PIN,LOW);

digitalWrite(LED_PIN,LOW);

}

}

}